Ganga River basin Details

| Parameters | Details | Reference |
| --- | --- | --- |
| Length | 2525 km | CWC (2014) |
| Catchment area | 861542 sq. km | CWC (2014) |
| Geographical area cover | 26.3% of the country’s total geographical area | Trivedi et al. (2010) |
| Lat long | spreads between 73° 2′ to 89° 5′ E and 21° 6′ to 31° 21′ N. | Trivedi et al. (2010) |
| Mean annual temperature | 24 °C The basin shows a rising trend with 0.04 °C/year in Nepal and 0.091 °C/year in Bangladesh, which is much higher than the increase in global average temperatures | Kirby et al. (2015) |
| Hydrology | The basin receives nearly 80% of its total rainfall in the monsoon period of June–October and its average annual rainfall varies from 400 to 2000 mm. The average annual rainfall in the Indian part of Ganga varies from less than 400 mm in the western regions, 1,200 mm in the middle to more than 2,000 mm in the eastern parts. | Surinaidu et al. (2020) Zakwan et al. (2021) |
| Topography | Mountains dominate the north with at 8,848 meters (AMSL). The flat plains, at less than 100 m AMSL, dominate the middle part of the basin. | GOI (2014), |
| Geology | The basin is divided from north to south into the Himalayan region, sub-Himalayan region, Bhabar zone, Terai zone, central Ganga plains, and marginal alluvial plains. The aquifer system's complexity is driven by depositional features and geomorphology. | Surinaidu et al (2020) |
| Hydrogeology | The basin features thick alluvial sediments deposited by the river and its distributaries, forming unconfined and leaky aquifers that extend from 1,500 to 2,000 m below ground level. These aquifers consist of sand, gravel, pebbles, and multiple high-yielding layers. | MacDonald et al. (2016) |
| Basin Cover Different States | Uttar Pradesh, (240,798 km2), Uttarakhand (53,566), Bihar (143,961 km2), West Bengal (71,485 km2) and Delhi (1,484 km2), and part of Madhya Pradesh (198,962 km2), Rajasthan (112,490 km2), Haryana (34,341 km2) and Himachal Pradesh (4,317 km2) | Trivedi et al. (2010) |
| Tributaries | the Kali, the Ramganga, the Yamuna, the Gomti, the Ghaghara, the Gandak and the Kosi, the Tons, the Son, the Damodar and the Kasai-Haldi, Yamuna. | Trivedi et al. (2010) |
| Soil type | Predominant soil types found in the basin are sand, loam, clay and their combinations, such as sandy loam, loam, silty clay loam and loamy sand soils. | Misra (2011) |
| Soil Erodibility | In the Ganga basin, soil erodibility varies: - Mountain, submontane, and alluvial soils (58%) are highly erodible. - Red soils (12%) have high erodibility. - Red & yellow, mixed red & black soils (8%) show moderate erodibility. - Deep black and medium black soils (14%) have low erodibility. - Shallow black and lateritic soils (6%) have very low erodibility. | https://nmcg.nic.in/soil.aspx |

References

- GoI (Government of India) 2014 Ganges Basin, Central Water Commission. Ministry of Water Resources, New Delhi, India.

- Indiawaris https://indiawris.gov.in/downloads/Ganga%20Basin.pdf

- Kirby, J. M., Ahmad, M. D., Mainuddin, M., Palash,W., Quadir, M. E., Shah-Newaz, S. M. & Hossain,M.M. 2015 The impact of irrigation development on regional groundwater resources in Bangladesh. Agricultural Water Management 159, 264–276.

- MacDonald, A. M., Bonsor, H. C., Ahmed, K. M., Burgess, W. G., Basharat, M., Calow, R. C., Dixit, A., Foster, S. S. D., Gopal, K., Lapworth, D. J., Lark, R. M., Moench, M., Mukherjee, A., Rao, M. S., Shamsudduha, M., Smith, L., Taylor, R. G., Tucker, J., van Steenbergen, F. & Yadav, S. K. 2016 Groundwater quality and depletion in the Indo-Gangetic Basin mapped from in situ observations. Nature Geoscience 9.

- Misra, A. K. (2011). Impact of urbanization on the hydrology of Ganga Basin (India). Water resources management, 25, 705-719.

- National Mission for clean ganga. https://nmcg.nic.in/soil.aspx

- Surinaidu, L., Amarasinghe, U., Maheswaran, R., & Nandan, M. J. (2020). Assessment of long-term hydrogeological changes and plausible solutions to manage hydrological extremes in the transnational Ganga river basin. h2oj, 3(1), 457-480.

- Trivedi, R. C. (2010). Water quality of the Ganga River–an overview. Aquatic Ecosystem Health & Management, 13(4), 347-351.

- Zakwan, M., & Ahmad, Z. (2021). Trend analysis of hydrological parameters of Ganga River. Arabian Journal of Geosciences, 14, 1-15.