Title: Restoration of Chromium Contaminated Soil-Groundwater Systems at Rania COPR Site, Kanpur Dehat, India

Background:

Observing the rapidly growing concern over groundwater contamination and deteriorating soil-water quality, we moved to Rania and Khan Chandpur villages in Kanpur Dehat district in Uttar Pradesh. Rania-Khan Chandpur site is one of the highly contaminated sites in India (Prakash et al. 2011; CPCB, 2023) due to Chromite Ore Processing Residue (COPR), which is a hazardous (waste) remnant of the chromate (CrO42-) extraction roasting process (Földi et al. 2013; Matern et al. 2017; 2020). As per report of the oversight committee, Hon’ble NGT, U.P. Lucknow, there is an estimated volume of 1,22,800 m3 COPR at Rania site (which has recently relocated to the treatment, storage, and disposal facility (TSDF) located 25Km from this site near Akbarpur, Kanpur Dehat). Both the villages are settled in the vicinity of the leather processing clusters and COPR waste from these tanneries and related industries has severely contaminated soils, the surface-water, and the groundwater. The residents are drinking Cr-containing groundwater and are always at risk through the poisonous intake either through groundwater or locally grown vegetables or other sources (Gupta et al. In Preparation). Regardless of this situation, the remediation praxis of Cr-contaminated soil-water systems remain unattempt at the Rania-Khan Chandpur, this is because the site-scale Cr fate and mobility in the subsurface is still less investigated.

Cr leachate from the COPR can infiltrate in the vadose zone and ultimately meet the (shallow-) groundwater which can result as huge Cr contamination in the subsurface. Our initial (Gupta et al. 2023a; Deoli et al. 2023) and other’s works (Singh et. al., 2009; Matern et. al., 2017) have found that groundwater, surface-water, and soil samples from the past have shown signs of a Cr emanating from the COPR dump site in and around Rania-Khan Chandpur villages. In the subsurface, Cr can be in the form of Cr (III) or Cr (VI), the mechanisms and kinetics of Cr release are obscure (Bhattacharya et. al., 2019). The release of Cr from the COPR dump site can be influenced by factors such as the pH of the water/soil, the presence of other chemicals or ions, the hydrogeology and hydrology of the site, the above and below-ground ecology and microbial community dynamics, and so on. Further, the climate change may influence the fate and mobility of the Cr by altering the hydro-metrological and subsurface conditions. A better understanding is needed to improve remediation practices including evaluation of potential Cr transport pathways and rates, and more efficient clean-up strategies. However, there remains a dearth of knowledge on Cr fate, transport, and remediation in the soil-water system.

Considering the research gaps prior to the implementation of this research work, a series of investigations has been initiated to generate baseline data of soil-water quality at the Rania-Khan Chandpur site. This has resulted in basic knowledge of the initial site conditions and profusion of excellent baseline data facilitate a better understanding of the soil-water system at this site. In this research work, the fate and mobility of Cr in the subsurface under varying hydrogeological conditions will be investigate first. Thereafter, the performance of the remediation techniques will be evaluated for restoration of Cr-contaminated soil-water systems at a demonstrative site in Rania-Khand Chandpur villages. This project will provide very accurate, multimethod techniques for the forecasting of the Cr mobility in the subsurface, which can assist the site managers/stakeholders for the future site scale planning. Further, this project will not only improve our understanding to manage a highly contaminated site in India, but also helps in the management of such contaminated sites in other part of the globe.

Motivation of the Study:

We moved to Rania-Khan Chandpur villages in the Kanpur Dehat district of Uttar Pradesh to investigate the ground zero-condition and have found that the soil and groundwater contamination have direct human health and ecological consequences. The entire village is living with several health issues such as severe skin disorders, gastrointestinal infection, heartburn, etc. eventually this soil-groundwater pollution has affected various components of the ecosystem. Hence, the urgent need for effective restoration measures and the lack of comprehensive studies in this specific context have motivated this research proposal.

Work novelty/Research Gaps:

A little information is available on the Cr mobility in the soil-water system (Gupta et al. 2023b). No effort has been initiated yet to explore the possibility of (bio-)remediation of Cr contaminated soil and groundwater resources in and around Rania-Khan Chandpur COPR site. Therefore, this proposal will investigate the site-scale Cr mobility in the subsurface under varying hydrogeological conditions. This proposal is novel as:

1. This research proposal will be the first to provide a series of precise investigations to model the fate, and mobility of Cr in soil-groundwater systems at the Rania COPR site. Which, as far we known, will be the first of its kind study at Indian contaminated site.

2. This work will develop a multiscale Cr forecasting model/tool/ approach that can account for the complex interactions between Cr mobility, hydrogeological conditions, and potential climate change impacts.

3. This project not only evaluate the performance of potential remediation techniques including Permeable Reactor Barrier (PRB) for Cr-contaminated groundwater, but also help to improve clean-up strategies for soil-surface water systems.

Research Objectives:

1. To investigate the fate and transport of Cr in soil-groundwater systems using a multiscale approach under varying subsurface conditions.

2. To investigate Cr mobility in the subsurface and forecast the future Cr-contamination loads in and around COPR contaminated Rania-Khan Chandpur site.

3. To design remediation techniques for Cr contaminated soil-groundwater systems and evaluate the performance of such techniques including PRB for the Cr removal from polluted groundwater at Rania site.

Methodology:

The proposed research programme aimed to develop a multiscale approach to investigate the Cr-contaminated Rania-Khan Chandpur site underpinning of the laboratory investigation of the fate and transport of Cr and co-contaminants in the site’s hydrogeological formation. The entire investigation projected in three major phases, in first phase the laboratory simulation will conduct for gathering the required data and information and in second phase, the field and modelling work will be finalised to upscale the fate and transport mechanisms under the real site hydrological and under the future climatic conditions. Later, based on the finding of the laboratory, field, and modelling works, the designs of the (bio)remediation techniques including PRB will be tested.

Work Plan#1: Laboratory and numerical modelling to mimic Rania-Khan Chandpur site conditions:  A series of 2-D and 3-D scale experiment will be performed to mimic the real site hydrogeological conditions to characterise the Cr fate and transport of parameters using undisturbed cores collected from the Rania/Khand Chandpur. Two-dimensional tank has already been fabricated as designed by Gupta et al. (2019) for this purpose. Three-dimensional tank will be fabricated as designed by Gupta and Yadav (2020). The main body is constructed tank of one piece of 1.5 mm thick stainless steel formed into a box with inner dimensions of 100cm-long x 100cm-width x100cm-deep. The front cover is made of a thick sheet and the space between the two walls is packed with undisturbed cores. The groundwater will be flowing horizontally from left to right controlled by two end chambers. The spatial and the temporal distributions of the Cr and its concentration in soil-water phase along with soil moisture content will be observed using Liquid chromatography–mass spectrometry (ICMS) and time domain reflectometers (TDRs).

Initially, sample will be characterized for their hydrophysical (Porosity, Bulk Density, Ksat, VGM parameters, soil moisture retention, etc.) and chemical properties, (pH, organic matter content, Cr and other hazardous materials through applications of synchrotron X-ray/optical methods). The collected cores will be transfer to the 2-D and 3-D glass tanks inbuild with TDRs and other sensors, which will be placed on Lysimeter for water budget estimation and attached with auxiliary column to maintain the groundwater flow/fluctuation dynamics. Multiple monitoring wells will be installed at different length and depths to collect soil-water samples. Soil-water samples will be collected with the target time interval to quantify the Cr and co-pollutants concentrations along with the pore water CH4 and CO2 as well asl atmospheric flux.

The water movement in the tank’s setups will be modelled using the Richard’s equation with HYDRUS 2/3D software. In porous media, the flow and transport are modelled as nonlinear coupled partial differential equations. The equations of unsaturated flow and solute transport are as follows:

∂θ∂t-∂K∂θ∂θ∂z-∇Dθ∇θ=0			(1)

∂(θC)∂t+q.∇C-∇.(DC∇C)=RC		(2)

Here (x,t) and C(t, x) are respectively the moisture content and solute concentrations. Cr will be main solute, to be solved using reactive transport model PHREEQC which is an integral part of HYDRUS software.

Both the unsaturated hydraulic conductivity K and moisture diffusivity D are nonlinear function of . The flux vector q= -∇-Kz( ),  is capillary pressure head. The reaction term in transport RC is often a nonlinear function of the solute concentration. During multi-species transport process, C becomes a vector to represent the concentration of all species and reaction term becomes a function all species concentrations. Initial and background conditions will be incorporated based on the observations from the current Cr concentration and hydrological conditions at Rania-Khand Chandpur site.

Outcomes of work plan#1: Soil moisture flow, Cr fate and transport parameters, model for upscaling the Cr mobility at site scale, a better understanding of role of the subsurface conditions on Cr fate and transport.

Work Plan#2: Site-scale Cr mobility model to predict and to forecast future contamination loads: This stage of the project will involve site-scale quantification of the hydrological fluxes and hydrogeology variables (for example, groundwater flow) to model Cr mobility in and around Rania-Khand Chandpur. First, the water budget component will be analysed by performing hydrological data collection (precipitation (P), evapotranspiration (ET), etc.) to understand the dominating water fluxes (P vs ET) and to calculate the fluxes in/out from the site-scale soil-water system. A site-scale numerical domain will be build using MODFLOW (USGS) to mimic hydrogeological formation of the Rania-Khand Chandpur using the geophysical data which has been already collected by my research group. The Cr mobility will be solved using the MODFLOW, in which the laboratory investigated the fate and transport parameters will be incorporated to run reactive flow of Cr. To validate the outcomes of modelling, the groundwater Cr concentrations and other geochemical variables will be used. Thereafter, the current and future climatic scenarios (data from CMIP5/CMIP6, https://esgf-node.llnl.gov/projects/cmip6/) will be added in the model to forecast the Cr contamination loads in the forthcoming years.

Outcomes of work plan#2: Cr fate and transport model to accurately predict and forecast Cr contamination loads at Rania-Khan Chandpur site.

Work Plan#3: Test for a demonstrative scale remediation of Cr and for performance evaluation of PRB at site: Based on the outcomes of work plan#1 and work plan#2, first, the same MODFLOW model will be incorporated with different (bio-)remediation conditions to explore (numerically) the possibility of a demonstrative scale test at real site. In this process, a highly reactive PRB materials will be placed in and around Cr source zone under site hydrogeological conditions to optimise the design of the proposed PRB. Effectiveness of the PRB will be optimise using laboratory tested materials (for example, biochar based PRBs). Likewise, other remediation approaches i.e., phytoremediation, bacterial remediation, etc. will be simulated by solving the decay/biodegradation kinetics along with the governing solute transport mechanisms. This numerical step will assist the demonstrative scale remediation of the Cr in soil-water system at Rania-Khan Chandpur. Finally, a series of demonstrative scale remediation activities will be implemented in Rania-Khan Chandpur and the effectiveness will be measured for long term. The design, size, and other operational variables of such demonstrative scale remediation will be finalised based on the outcome of modelling works and will be improved time-to-time based on the post-implementation observations.

Outcomes of work plan#2: A multimethod approach to treat Cr contaminations at Rania-Khan Chandpur site.

Outcomes of the study

This work will help local communities to make informed decisions about drinking water resource management, land use management, and infrastructure development to minimize exposure to such hazardous contaminant. This will also support the implementation of low cost commercial site scale measures to restore such a highly contaminated site, ensuring safe drinking water for the local population.

Target Beneficiaries

The target beneficiaries of this research will be the local community living near the Rania COPR Site, who will benefit from improved understanding of chromium contamination, potential health risks, and the implementation of effective restoration techniques. Furthermore, the regulatory/environmental authorities (like the CPCB/UPPCB) and other policymakers will benefit from the research results to develop strategies for contamination management and protection of soil-groundwater systems. The scientific community will acquire important new information on the fate, transport, and remediation techniques of chromium, expanding their understanding of environmental remediation. Considering India has growing environmental restoration marketplace of $100 billion; this study will also help future startups to explore the commercial scale restoration and remediation technologies (Mahajan et al. 2022).

References

Bhattacharya, A., Gupta, A., Kaur, A., and Malik, D. (2019). Alleviation of hexavalent chromium by using microorganisms: insight into the strategies and complications. Water Science and Technology, 79(3), 411-424.

Central Pollution Control Board, (2023). “Status of Contaminated Sites in India” Access at  https://cpcb.nic.in/overall-status/, dated 09/01/2023.

Deoli, V., Gupta, P. K., Dalvi, V., Nair, V., Malik, A. and Pant, K. K. (2023). “Vadose zone chromium distributions in samples collected from Rania-Khan Chandpur-Kanpur, India”. presentation in Japan Geoscience Union 2023, Chiba, Japan 21-26 May 2023.

Földi, C., Dohrmann, R., Matern, K., and Mansfeldt, T. (2013). Characterization of chromium-containing wastes and soils affected by the production of chromium tanning agents. Journal of Soils and Sediments, 13, 1170-1179.

Gupta, P. K., and Yadav, B. K. (2020). Three-dimensional laboratory experiments on fate and transport of LNAPL under varying groundwater flow conditions. Journal of Environmental Engineering, 146(4), 04020010.

Gupta, P. K., Nair, V., Dalvi, V., Dhali, S., Malik, A., and Pant, K. K. (2023a) Multi-disciplinary approach to understand a Cr contaminated site at Rania-Khan Chandpur, India. presentation in Japan Geoscience Union 2023, Chiba, Japan 21-26 May 2023.

Gupta, P. K., Yadav, B., and Yadav, B. K. (2019). Assessment of LNAPL in subsurface under fluctuating groundwater table using 2D sand tank experiments. Journal of Environmental Engineering, 145(9), 04019048.

Gupta, P.K., Nair, V., Dalvi, V., Dhali, S., Malik, A., Pant, K.K. (2023b). “Fate, transport, and bioremediation of Chromium in soil-groundwater system at Rania-Khan Chandpur site, India”, Goldschmidt, Lyon, to be held July 9-14, 2023.

Gupta, P.K., Nair, V., Dalvi, V., Dhali, S., Malik, A., Pant, K.K., (In preparation). Plant and soil microbial community assessment in and around Rania – Khan Chandpur Cr Contaminated Site, India. to be submitted to Journal of Hazardous Materials.

Mahajan, M., Gupta, P. K., Singh, A., Vaish, B., Singh, P., Kothari, R., & Singh, R. P. (2022). A comprehensive study on aquatic chemistry, health risk and remediation techniques of cadmium in groundwater. Science of The Total Environment, 818, 151784.

Matern, K., Weigand, H., Kretzschmar, R., and Mansfeldt, T. (2020). Leaching of hexavalent chromium from young chromite ore processing residue 49(3), 712-722.

Matern, K., Weigand, H., Singh, A., and Mansfeldt, T. (2017). Environmental status of groundwater affected by chromite ore processing residue (COPR) dumpsites during pre-monsoon and monsoon seasons. Environmental Science and Pollution Research, 24, 3582-3592.

Prakash, R., Srivastava, S. K., Singh, M., Rastogi, R., Singh, K., and Bhartariya, K. G. (2011). Metallic contamination in ground water of Rania industrial area, Kanpur, UP. In National Workshop on chemistry our life, our future. Lucknow, India.

Singh, R., Misra, V., and Singh, R. P. (2009). Synthesis, characterization, and role of zero-valent iron nanoparticle in removal of hexavalent chromium from chromium-spiked soil. Journal of Nanoparticle Research, 13, 4063-4073.